L10: WINDOWS / ROOFLIGHTS / BRISE SOLEIL / LOUVRES.

The following specification shall be read in conjunction with the Architect’s (………) Series drawings:

ALUMINIUM BRISE SOLEIL:

- General:

  - Where clearly indicated on Architect’s drawings, the solar shading attachments shall be manufactured from Metal Technology’s Aluminium Brise Soleil System.

  - The system shall be offered in a single polyester powder coated colour finish. Supporting outriggers are attached back to the curtain walling structure by means of Metal Technology’s bespoke thermally broken mounting bracket.

  - The Brise Soleil system shall be offered with aerofoil fixed between the outriggers subject to site conditions.

- System Supplier:

  - Metal Technology Ltd., Steeple Road Industrial Estate, Steeple Road, Antrim, Co. Antrim, Northern Ireland, BT41 1AB.  Tel: 028 9448 7777.

  - Email: sales@metaltechnology.com or Web: www.metaltechnology.com

- Accreditation:

  - The Systems Company is to have ISO 9001 Quality Management Standard, and ISO 14001 Environmental Management Systems Standard.

- Installation Partners:

  - A list of specific and approved fabrication partners for this installation can be made available to the main contractor on request (Please contact your Metal Technology Regional Manager for details).

  - The fabricator shall make provision for producing construction drawings for approval prior to commencing works.

- Installation:

  - The ‘CWCT Standard for Systemised Building Envelopes’ is to be used as a reference for all projects containing curtain walling/windows/doors.

  - All glazed elements within this package should be installed by experienced and qualified personnel, possessing a recognised installer’s qualification.

- Product Reference:

  - System 22 Brise Soleil.

- Materials:

  - All profiles to be extruded from aluminium alloy 6060 T6, T5 or T4 to BS EN 12020-2; 2001 / BS 755-9; 2001.

- Finish:

  - The finish to all aluminium profiles shall generally be Polyester Powder Coated to BS EN 12206 in a standard RAL colour to satisfy a corrosion classification as defined by the powder manufacturer.

  - Powder coat shall be applied to an average thickness of 60 microns.

  - All powder coating shall be supported by a manufacturers/applicators 20 year guarantee for a standard grade finish subject to the applicators approval.

  - Project requirements must be clearly identified by the design team responsible.

  - Finish:         			Colour Ral ……..

- Construction:

  - The Brise Soleil shall be manufactured and installed in strict accordance with Metal Technology’s instructions and guidelines as set down in their technical literature, details and specifications. The Brise Soleil should be installed with a horizontal projection and fitted directly into the curtain walling structure by means of specially fabricated mounted brackets (S70). Side arms (BS36) shall be hooked onto prepositioned pins contained within the fixing brackets and bolted into position. Between the projecting side arms Metal Technology’s 200mm blade (BS06) shall be square cut and fixed at a 32.5° pitch and closed off at the front with nosing blade (BS16). All fixings shall be stainless steel.

- Fixing:

  - When fixed directly onto curtain walling or building structure, all fixings shall be in strict accordance with Metal Technology’s instructions and guidelines as detailed in their technical literature.

  - Details/evidence of all calculations shall be submitted by the fabricator for approval prior to commencing installation.

  - All fixing shall conform to BS 6262.

  - All outriggers shall be fixed securely back to the curtain walling mullion box by means of Metal Technology’s bespoke thermally broken mounting bracket.