L10: WINDOWS / ROOFLIGHTS / SCREENS / LOUVRES.

The following specification shall be read in conjunction with the Architect’s (……….) series drawings:

ALUMINIUM WINDOWS:

- General:

  - Where clearly indicated on Architect’s schedules and drawings, all windows and window inserts shall be manufactured from Metal Technology’s Thermally Enhanced Casement Window System.

  - The system is available in a single or dual colour polyester powder coated finish and shall be able to contain double glazed units and inserts as described in their appropriate specification.

  - The enhanced thermal performance of the windows, screens, doors and their respective inserts, in conjunction with the appropriate glazing specification, shall be capable of meeting the current approved Building Regulations.

- System Supplier:

  - Metal Technology Ltd., Steeple Road Industrial Estate, Steeple Road, Antrim, Co. Antrim, Northern Ireland, BT41 1AB.  Tel: 028 9448 7777.

  - Email: sales@metaltechnology.com or Web: www.metaltechnology.com

- Accreditation:

  - The Systems Company is to have ISO 9001 Quality Management Standard, and ISO 14001 Environmental Management Systems Standard.

- Installation Partners:

  - A list of specific and approved fabrication partners for this installation can be made available to the main contractor on request (Please contact your Metal Technology Regional Manager for details).

  - The fabricator shall make provision for producing construction drawings for approval prior to commencing works.

- Installation:

  - The ‘CWCT Standard for Systemised Building Envelopes’ is to be used as a reference for all projects containing curtain walling/windows/doors.

  - All glazed elements within this package should be installed by experienced and qualified personnel, possessing a recognised and relevant installer’s qualification.

- Product Reference:

  - System 4-20 Thermally Enhanced Polyamide Casement Window.

- Materials:

  - All profiles to be extruded from aluminium alloy 6060 T6, T5 or T4 to BS EN 12020-2 / BS 755-9 and with polyamide thermal break sections manufactured from glass reinforced nylon sections capable of withstanding temperatures in excess of 200 ºC.

- Finish:

  - The finish to all aluminium profiles shall generally be Polyester Powder Coated to BS EN 12206 in a standard RAL colour to satisfy a corrosion classification as defined by the powder manufacturer.

  - Powder coat shall be applied to an average thickness of 60 microns.

  - All powder coating shall be supported by a manufacturers/applicators 20 year guarantee for a standard grade finish subject to the applicators approval.

  - Project requirements must be clearly identified by the design team responsible.

  - Internal Finish:         		Colour Ral ……..

  - External Finish:			Colour Ral ……..

- Performance:

  - The windows to provide Class IV weather tightness as defined by BS 6375 Part 1 (Air 600 Pa class IV, Water 600 Pa, and Wind 2400 Pa).

- Exposure:

  - Design Wind Pressure to be determined in accordance with BS 6399 - Part 2.

- Thermal Performance:

  - All glazed curtain walling, windows, doors and their respective inserts, in conjunction with the appropriate glazing specification, should be capable of meeting the current approved Building Regulation.

  - Weighted Average U-value:		………..W/m²K.

  - Details/evidence of all calculations/methods of determining the required project average U-value for the curtain walling screens and their respect inserts shall be submitted by the fabricator with all tender submissions for approval by the project Architect.

- Secured by Design:

  - All glazed curtain walling screens, windows, and doors within the ground floor and vulnerable areas shall embrace secure by design.

  - All systems installed must demonstrate that they have been tested in line with PAS24 security standards and certified by a UKAS accredited certification body.

- Structure:

  - All structural profiles to be designed so as the maximum deflection of any member shall not exceed L/175 of its span with any evidence of any permanent deformation once the load has been removed.

  - All vents shall be sized in accordance with the guidelines detailed in Metal Technology’s technical literature.

  - All structural profiles’ stability and suitability must be confirmed by the fabricators engineers.

  - Details/evidence of all calculations shall be submitted by the fabricator with for approval prior to commencing installation.

- Construction:

  - All windows shall be manufactured, installed and glazed in strict accordance with Metal Technology’s instructions and guidelines as set down in the appropriate technical literature.

  - All opening windows shall perform with a top hung or side hung open out operation.

  - Minimum depth of outer frame sections shall be 50mm incorporating two 20mm polyamide thermal break sections within the window profiles.

  - Windows should be coupled as necessary.

  - All outer frame and vent frame members shall be 45° mitred corner construction and reinforced by means of extruded aluminium cleats with stainless steel corner braces.

  - The corner joints shall be pneumatically crimped and secured with a two part adhesive.

  - All mullions transoms shall be square cut, shaped and secured using suitable stainless steel screws driven into integral screw ports within the sections where required.

  - Cruciform cleats to be used were required.

  - The “T” joints should be sealed during construction using suitable ‘small gap’ sealant.

  - All gasket joints within the frame must be sealed using a suitable sealant.

  - The windows shall incorporate an internal pressure equalised drainage system with external snap on drainage cover caps.

  - All windows shall be designed and manufactured in compliance with the relevant British Standards.

- Liners Details (Option A):

  - All windows shall incorporate cill liner profile to accommodate made to measure bespoke push fit aluminium cill pressing (please consult with your Metal Technology’s Architectural Advisor for available options).

- Liners Details (Option B):

  - All windows shall be fabricated using Metal Technology’s Panel Adaptor Profiles allowing easy integration within the proposed panel system.

- Glazing System:

  - All double glazed units, panels and inserts shall be supported continuously along all four edges by specialised glazing support blocks.

  - All windows shall be dry glazed using ‘snap in’ extruded aluminium beads.

  - Metal Technology’s standard, co-extruded PVC Nitrile captive ‘E’ gaskets externally and wedge gasket internally.

  - The corners of the gaskets shall be accurately mitred together and sealed to ensure an effective joint.

  - Setting blocks and location pieces shall be fitted in accordance with BS 6262 in order to ensure the windows are maintained square and rigid.

- Sealed Unit Details:

  - Where clearly indicated, all curtain walling screens, windows and doors shall contain sealed units, which shall be manufactured in accordance with BS EN 1279 Parts 2 and 3 and carry the necessary and appropriate guarantee.

  - Internal Pane:			…………………...

  - External Pane:			……………………

  - All glass units shall be gas filled with warm edge spacer with exact specification determined by the “U” Value calculation.

  - Where solar control glass is applicable:

  - Light Transmission: 			…….

  - G Factor:				…….

  - The Fabricator is to conduct a thermal safety check against glass fracture where required.

  - Manifestation as required.

  - All glazing shall conform to BS6262 and must be in compliance with the current Building Regulations.

- Panel Details (Option A):

  - Where clearly indicated, the panels shall generally be double glazed glass spandrel panels, which shall be manufactured in accordance with BS EN 1279 Parts 2 and 3 and carry the necessary and appropriate guarantee.

  - Internal Pane:			…………………...

  - External Pane:			……………………

  - Each panel shall be supplied complete with a foil backed insulation internally and where exposed, the internal face shall have polyester powder coated aluminium tray finish as required and in line with specification:

  - Internal Finish:         		Colour Ral ……..

  - All glass units shall be argon filled with warm edge spacer with exact specification determined by the “U” Value calculation.

  - The Fabricator is to conduct a thermal safety check against glass fracture where required.

  - All glazing shall conform to BS6262 and must be in compliance with the current Building Regulations.

- Panel Details (Option B):

  - Where clearly indicated, the dead panels within the screens shall generally be aluminium faced spandrel panels:

  - They shall be finished externally with a 2mm polyester powder coated aluminium sheet.

  - The powder coated finish to all external aluminium faces shall generally be a minimum thickness of 60 microns.

  - Each panel shall be supplied complete with a foil backed insulation internally and where exposed, the internal face shall have polyester powder coated aluminium tray finish as required and in line with specification:

  - Internal Finish:         		Colour Ral ……..

- All aluminium panels will be subject to confirmation by the design team and shall be in compliance with the current Building Regulations.

  - The exact specification will be determined by the “U” Value calculation.

- All glazing shall conform to BS 6262.

- Incorporated Components:

  - Where clearly indicated, each window shall incorporate a glazed-In ventilator.

  - It shall be polyester powder coated to match the window profiles.

  - They shall be fitted with an external, aluminium hood and shall be fitted with cord control.

  - Each vent must be capable of providing a minimum free airflow in accordance with the current Building Regulations.

  - Vents shall be tested to BS 6375 Part 1 1989, and achieve Category C – 600Pa wind & water tightness.

- Fixing:

  - All fixings shall be in strict accordance with Metal Technology’s instructions and guidelines as detailed in their technical literature and in accordance with the relevant British Standards, including BS6262.

  - Fixing shall ensure the window is retained securely within the opening without incurring any damage or distortion to the window frame.

  - Details of the proposed fixing method shall be submitted to the project engineer for approval prior to installation.

- Accessories:

  - All aluminium flashings, sills and other perimeter trims necessary to ensure the performance of the glazing system shall be the responsibility of the glazing sub-contractor.

  - They shall be designed, fabricated, finished, install, secured and sealed in accordance the architect’s details.

  - Glazing contractor to take full responsibility for butt straps and end caps to all pressings.

- Ironmongery Options:

  - We would recommend consultation between the Design Team and Metal Technology on ironmongery options available to ensure compatibility.

- Hinges:

- Stainless steel friction hinges all sized and fitted in accordance with Metal Technology’s technical literature.

- Restrictors;

- To meet the required security level and health and safety standard all opening vent frames shall be fitted with one pair of non-releasable or releasable locking restrictors.

- The window opening shall be restricted to 100mm or in line with the rapid ventilation calculations.

- These should be fitted in accordance with Metal Technology’s recommendations and as detailed in their technical literature.

- Window Handle Options:

- Metal Technology’s single handle espagnolette multi-point locking system.

- Metal Technology’s Window Pull Mechanism.

- Metal Technology’s Folding Opener.

- Fireman’s Axe Boss.

- Loose Handle for Fireman’s Axe Boss.

- Spring Catch and Link Rod.

- BS7950.

- Lifetime Homes.

- Window Gearing Option:

  - Where opening windows are considered to be unreachable they shall be fitted with a remote manual chain actuator.

  - The number of chain operators utilised will be determined by window vent width.

  - Each set of Actuators shall be controlled by a remote rotary control.

  - Individual windows or window sets shall be shown on the window schedule and confirmed by the architect.

- Window Gearing Option:

  - Each vent shall be fitted with an automated actuator capable of being incorporated within the building management system.

  - They shall utilise a sliding bracket along its body and shall have a noiseless operation.

  - They shall require a single phase voltage of 230V which shall be supplied by others.