L10: WINDOWS / ROOFLIGHTS / SCREENS / LOUVRES.

The following specification shall be read in conjunction with the Architect’s (……….) series drawings:

ALUMINIUM WINDOWS:

- General:

  - Where clearly indicated on Architect’s schedules and drawings, all windows and window inserts shall be manufactured from Metal Technology’s Thermally Broken Vertical Sliding Window.

  - The system shall be offered in a polyester powder coated colour finish and shall be able to contain double glazed units and inserts as described in their appropriate specification.

  - The enhanced thermal performance of the windows, screens, doors and their respective inserts, in conjunction with the appropriate glazing specification, shall be capable of meeting the current approved Building Regulations.

- System Supplier:

  - Metal Technology Ltd., Steeple Road Industrial Estate, Steeple Road, Antrim, Co. Antrim, Northern Ireland, BT41 1AB.  Tel: 028 9448 7777.

  - Email: sales@metaltechnology.com or Web: www.metaltechnology.com

- Accreditation:

  - The Systems Company is to have ISO 9001 Quality Management Standard, and ISO 14001 Environmental Management Systems Standard.

- Installation Partners:

  - A list of specific and approved fabrication partners for this installation can be made available to the main contractor on request (Please contact your Metal Technology Regional Manager for details).

  - The fabricator shall make provision for producing construction drawings for approval prior to commencing works.

- Installation:

  - The ‘CWCT Standard for Systemised Building Envelopes’ is to be used as a reference for all projects containing curtain walling/windows/doors.

  - All glazed elements within this package should be installed by experienced and qualified personnel, possessing either a recognised and relevant vocational qualification, or CWCT Window Installers qualification.

- Product Reference:

  - System 6 Thermally Broken Vertical Sliding Window.

- Materials:

  - All profiles to be extruded from aluminium alloy 6060 T6, T5 or T4 to BS EN 12020-2 / BS 755-9.

- Finish:

  - The finish to all aluminium profiles shall generally be Polyester Powder Coated to BS EN 12206 in a standard RAL colour to satisfy a corrosion classification as defined by the powder manufacturer.

  - Powder coat shall be applied to an average thickness of 60 microns.

  - All powder coating shall be supported by a manufacturers/applicators 20 year guarantee for a standard grade finish subject to the applicators approval.

  - Project requirements must be clearly identified by the design team responsible.

  - Finish:         			Colour Ral ……..

- Performance:

  - The windows to provide Class IV weather tightness as defined by BS 6375 Part 1 (Air 600 Pa class IV, Water 600 Pa, and Wind 2400 Pa).

- Exposure:

  - Design Wind Pressure to be determined in accordance with BS 6399 - Part 2.

- Thermal Performance:

  - All glazed curtain walling, windows, doors and their respective inserts, in conjunction with the appropriate glazing specification, shall be capable of meeting the current approved Building Regulation.

  - Weighted Average U-value:		………..W/m²K.

  - Details/evidence of all calculations/methods of determining the required project average U-value for the curtain walling screens and their respect inserts shall be submitted by the fabricator with all tender submissions for approval by the project Architect.

- Structure:

  - All structural profiles shall be designed so as the maximum deflection of any member shall not show any evidence of permanent deformation once the load has been removed in line with C.W.C.T. Clause 5.5.2.2:

  - 0≥3000mm = L/200

  - 3000m≥7500mm = L/300+5

  - All vents shall be sized in accordance with the guidelines detailed in Metal Technology’s technical literature.

  - All structural profiles’ stability and suitability must be confirmed by the fabricators engineers. Details/evidence of all calculations shall be submitted by the fabricator with for approval prior to commencing installation.

- Construction:

  - All windows shall be manufactured, installed and glazed in strict accordance with Metal Technology’s instructions and guidelines as set down in the appropriate technical literature, details and specifications.

  - All opening windows shall perform with a vertical slide operation.

  - Minimum depth of outer frame sections shall be 85mm incorporating a 12mm resin thermal break section within the window profiles.

  - Windows should be coupled as necessary.

  - All joints to be generally shall be square cut, shaped and screwed using stainless steel screws driven into integral screw ports within the sections.

  - All joints and secured with a two part adhesive (MT1803) to increase the stability of the joints.

  - All mullions transoms shall be square cut, shaped and secured using suitable stainless steel screws driven into integral screw ports within the sections where required.

  - The lower sash to be sealed at the sill by means of a PVC Nitrile & bubble gasket held within an integral gasket groove extruded within the sill section.

  - All gasket joints within the frame must be sealed using a suitable sealant to prevent water ingress (HR50328).

  - The windows shall incorporate an internal pressure equalised drainage system with external snap on cover caps.

  - All windows shall be designed and manufactured in compliance with BS4873.

- Glazing System:

  - All double glazed units, panels and inserts shall be supported continuously along all four edges by specialised glazing support blocks.

  - All windows shall be dry glazed using external ‘snap in’ extruded aluminium beads.

  - All opening vents shall be dry glazed using Metal Technology’s standard “wrap-a-round” gasket.

  - Metal Technology’s standard colour coded, co-extruded PVC Nitrile captive ‘E’ gaskets externally and wedge gasket internally on all fixed lights.

  - The corners of the gaskets shall be accurately mitred together and sealed to ensure an effective joint.

  - Setting blocks and location pieces shall be fitted in accordance with BS 6262 in order to ensure the windows are maintained square and rigid.

- Sealed Unit Details:

  - Where clearly indicated, all curtain walling screens, windows and doors shall contain sealed units, which shall be manufactured in accordance with BS EN 1279 Parts 2 and 3 and carry the necessary and appropriate guarantee.

  - Internal Pane:			…………………...

  - External Pane:			……………………

  - All glass units shall be argon filled with warm edge spacer with exact specification determined by the “U” Value calculation.

  - Where solar control glass is applicable:

  - Light Transmission: 			…….

  - G Factor:				…….

  - The Fabricator is to conduct a thermal safety check against glass fracture where required.

  - Manifestation as required.

  - All glazing shall conform to BS6262 and must be in compliance with the current Building Regulations.

- Incorporated Components:

  - Where clearly indicated, these shall consist of Metal Technology’s Systems as detailed in their appropriate specification. (Please consult with Metal Technology for further recommendations and details).

- Fixing:

  - All fixings shall be in strict accordance with Metal Technology’s instructions and guidelines as detailed in their technical literature and in accordance with the relevant British Standards, including BS6262.

  - Fixing shall ensure the window is retained securely within the opening without incurring any damage or distortion to the window frame.

  - Details of the proposed fixing method shall be submitted to the project engineer for approval prior to installation.

- Accessories:

  - All aluminium flashings, sills and other perimeter trims necessary to ensure the performance of the glazing system shall be the responsibility of the glazing sub-contractor.

  - They shall be designed, fabricated, finished, install, secured and sealed in accordance the architect’s details.

  - Glazing contractor to take full responsibility for butt straps and end caps to all pressings.

- Ironmongery:

  - As required the window systems shall be manufactured supplied and installed in strict accordance within the guidelines laid down in Metal Technology’s Technical Literature..

  - The window systems shall have been proven to pass the relevant performance tests associated with the same.

  - Prior to specification, the design team should take into consideration the relevant design criteria laid out in the Building Regulations for the safe opening and closing of windows and natural ventilation.

  - Compatibility of the ironmongery with the choice of system is essential prior to tender.

  - Different ironmongery selections are available and should the project requirements differ from the standard recommended, Metal Technology strongly advises consultation between their local Architectural Advisor and the Design Team.

- Gearing:

  - All opening vents shall be fitted with unique spiral spring balances as supplied by Metal Technology.

  - Balances designed to support the sashes in any opening position all balances to be sized, installed & adjusted in accordance with the manufacturer’s recommendations for vent size and weight, etc.

- Locking:

  - All opening vents shall be secured at the meeting stiles using a standard Fitch lock and keep.

  - Windows in excess of 1000mm wide to incorporate 2 No. sash locks.