L20: DOORS / SHUTTERS / HATCHES

The following specification shall be read in conjunction with the Architect’s (…………) Series drawings and in conjunction with H11 and L10 of the specification:

200: EXTERNAL ALUMINIUM ENTRANCE DOORS:

- General:

  - As clearly indicated, the external door sets shall be manufactured from Metal Technology’s “Thermal” Commercial Door Suite.

  - The external doors shall be offered as single, double action or automated.

  - The system is available in a single or dual colour polyester powder coated finish and shall be able to contain double glazed units and inserts as described in their appropriate specification.

  - The enhanced thermal performance of the windows, screens, doors, and their respective inserts, in conjunction with the appropriate glazing specification, shall be capable of meeting the current approved Building Regulations.

  - The door system shall have been proven to pass the relevant performance tests associated with the same.

  - Where the ironmongery selection differs from that used by Metal Technology within their testing procedures then the door leaves shall be deemed as non-compliant.

- System Supplier:

  - Metal Technology Ltd., Steeple Road Industrial Estate, Steeple Road, Antrim, Co. Antrim, Northern Ireland, BT41 1AB.  Tel: 028 9448 7777.

  - Email: sales@metaltechnology.com or Web: www.metaltechnology.com

- Accreditation:

  - The Systems Company is to have ISO 9001 Quality Management Standard, and ISO 14001 Environmental Management Systems Standard.

- Installation Partners:

  - A list of specific and approved fabrication partners for this installation can be made available to the main contractor on request (Please contact your Metal Technology Regional Manager for details).

  - The fabricator shall make provision for producing construction drawings for approval prior to commencing works.

- Installation:

  - The ‘CWCT Standard for Systemised Building Envelopes’ is to be used as a reference for all projects containing curtain walling/windows/doors.

  - All glazed elements within this package should be installed by experienced and qualified personnel, possessing either a recognised Installer’s qualification.

- Product References:

  - System 10-35Hi Thermally Enhanced Commercial Doors.

- Materials:

  - The minimum depth of door profiles shall be 62mm incorporating two 35mm polyamide thermal break sections within.

  - All profiles to be extruded from aluminium alloy 6060 T6, T5 or T4 to BS EN 12020-2 / BS 755-9.

  - Polyamide thermal break sections shall be manufactured from glass reinforced nylon sections capable of withstanding temperatures more than 200 ºC.

- Finish:

  - The finish to all aluminium profiles shall generally be Polyester Powder Coated to BS EN 12206 in a standard RAL colour to satisfy a corrosion classification as defined by the powder manufacturer.

  - Powder coat shall be applied to an average thickness of 60 microns.

  - All powder coating shall be supported by a manufacturers/applicator guarantee for a standard grade finish subject to the applicator’s approval.

  - Project requirements must be clearly identified by the design team responsible.

  - Internal Finish:      		   		Colour Ral ……...

  - External Finish:      		   		Colour Ral ……...

- Exposure:

  - Design Wind Pressure should be determined in accordance with BS6399 - Part 2.

- Thermal Performance:

  - All glazed curtain walling, windows, doors and their respective inserts, in conjunction with the appropriate glazing specification, shall be capable of meeting the current approved Building Regulation.

  - Average Weighted U-value:		…………W/m²K.

  - Details/evidence of all calculations/methods of determining the required project average U-value for the curtain walling screens and their respect inserts shall be submitted by the fabricator with all tender submissions for approval by the project Architect.

- Secured by Design:

  - All glazed screens, windows and doors within the ground floor and vulnerable areas shall embrace secure by design.

  - All systems installed must demonstrate that they have been tested and passed the PAS24 security standards and certified by a UKAS accredited certification body.

- Structure:

  - All structural profiles to be designed so as the maximum deflection of any member shall not exceed L/175 of its span with any evidence of any permanent deformation once the load has been removed.

  - All vents shall be sized in accordance with the guidelines detailed in Metal Technology’s technical literature.

  - All profiles should be able to withstand the stated wind load.

  - Structural profile stability and suitability must be confirmed by the appointed fabricator’s engineers.

  - Details/evidence of all calculations shall be submitted by the fabricator with for approval prior to commencing installation.

  - Any additional supporting structure required by the glazing contractor should be clearly identified within his tender.

  - Additional supporting structure identified shall be by others.

- Construction:

  - Door sets shall open out or open in as indicated on the schedules and floor plans.

  - The minimum depth of outer frame sections shall be 62mm incorporating two 35mm polyamide thermal break sections within the door profiles.

  - All outer frame members shall have a square cut corner construction and reinforced by means of extruded aluminium cleats.

  - The corner joints shall be screw fixed and secured with a two-part adhesive to increase the stability of the joints.

  - All joints should be sealed during construction using suitable ‘small gap’ sealant.

  - Door mid rails, cill rails and thresholds to be square cut, shaped and secured using extruded cleats, bolts and stainless-steel screws driven into integral screw ports within the sections.

  - All bottom rails shall be fitted with an appropriate drip rail in line with Metal Technology’s technical manual.

  - All external drainage weep holes to be protected and concealed behind snap on drainage cover caps.

  - Doors can be fitted with an anti-finger trap facility.

- Thresholds:

  - In commercial applications, Metal Technology’s thermally broken semi-rebated low-rise ramped threshold detail shall be utilised.

  - All threshold bars shall be suitable for wheelchair access.

- Glazing System:

  - All double-glazed units, panels and inserts shall be supported continuously along all four edges by specialised glazing support blocks.

  - All doors shall be dry glazed using external ‘snap in’ extruded aluminium beads.

  - Metal Technology’s standard co-extruded PVC Nitrile captive ‘E’ gaskets shall be used externally with wedge gasket internally.

  - The corners of the gaskets shall be accurately mitred together and sealed to ensure an effective joint.

  - Setting blocks and location pieces shall be fitted in accordance with BS 6262 to ensure the doors are maintained square and rigid.

- Sealed Unit Details:

  - Where clearly indicated, the curtain walling screens shall contain sealed units, which shall be manufactured in accordance with BS EN 1279 Parts 2 and 3 and carry the necessary and appropriate guarantee.

  - All seals to double glazed units must be “UV” resistant.

  - Internal Pane:			……………….

  - External Pane:			……………….

  - In areas of risk glass must comply with Parts K and M of the current building regulations and to be able to withstand expected site wind loadings.

  - Indicative barrier loading:		……………….

  - All glass units shall be argon filled with a warm edge spacer with the exact specification determined by the “U” Value calculation.

  - All toughened glass to be heatsoak tested with the chosen fabricator to conduct a thermal safety check against glass fracture where required.

  - When required the glass units shall generally satisfy the secure by design remit in ground floor and vulnerable areas.

  - Manifestation as required and where indicated.

  - All glazing shall conform to BS6262 and must comply with the current Building Regulations.

- Solar Control:

  - Where solar control glass is applicable:

  - Light Transmission: 			……………….

  - G Factor:				……………….

  - All figures subject to confirmation by M&E.

- Panel Details:

  - Where clearly indicated, the dead panels within the doors shall generally be aluminium faced insulated spandrel panels:

  - They shall be finished externally with a 2mm polyester powder coated aluminium sheet.

  - Powder coat shall be applied to an average thickness of 60 microns.

  - All powder coating shall be supported by a manufacturers/applicator guarantee for a standard grade finish subject to the applicator’s approval.

  - Project requirements must be clearly identified by the design team responsible.

  - Internal Finish:      		   		Colour Ral ……...

  - External Finish:      		   		Colour Ral ……...

  - All powder coating to match the curtain walling, window and door profiles.

- Louvres Details:

  - Where clearly indicated, the louvre panels contained within the curtain walling shall be manufactured from Metal Technology’s “Glazed in Louvre” System 23.

  - The outer frame members shall be designed to glaze directly into Metal Technology’s door system.

  - Size limitations should be in strict accordance with Metal Technology’s technical manual.

  - The louvre blades shall be retained within the panel at a pitch of either 33mm or 50mm subject to confirmation from the design team.

  - Each louvre panel installed shall incorporate a fly screen mesh or bird mesh.

  - The finish to all aluminium profiles shall generally be Polyester Powder Coated to BS EN 12206 in a standard RAL colour to satisfy a corrosion classification as defined by the powder manufacturer.

  - Powder coat shall be applied to an average thickness of 60 microns.

  - All powder coating shall be supported by a manufacturers/applicator 20 year guarantee for a standard grade finish subject to the applicators approval.

  - Project requirements must be clearly identified by the design team responsible.

  - Finish:         			Colour Ral ……...

  - All powder coating to match the curtain walling, window, and door profiles.

- External Door Ironmongery:

  - All opening leaves in conjunction with the selected ironmongery shall have been proven to pass the relevant performance tests associated with the same.

  - Prior to specification, the design team should take into consideration the relevant design criteria laid out in the Building Regulations for the safe opening and closing of doors.

- Locking Mechanisms:

  - Standard hook lock

  - Panic Mechanism.

- Door Closer Option:

  - Concealed.

  - Surface Mounted.

  - Automated.

- Handle Options:

- Metal Technology’s standard aluminium pad handle.

- Stainless steel pull handles.